### Supplementary Material, Table 1. Calibrated model parameters and values for Sprague River tributaries and mainstem (“Main”).

| Name | Description | File | Units | North Fork | South Fork | Sycan | Main |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ADJ_PKR | Peak rate adjustment factor for sediment routing in tributary channels. | .bsn | - | 1.768 | 0.705 | 0.500 | 0.710 |
| AI0 | Ratio of chlorophyll-a to algal biomass. | .wwq | μg-chla/mg algae | 10.070 | 0.588 | 1.000 | 17.500 |
| AI1 | Fraction of algal biomass that is N. | .wwq | mg N/mg algae | 0.089 | 0.070 | 0.090 | 0.090 |
| AI2 | Fraction of algal biomass that is P. | .wwq | mg P/mg algae | 0.011 | 0.010 | 0.010 | 0.010 |
| ALPHA_BF | Base flow alpha factor. | .gw | days | 0.001 | 0.033 | 0.048 | 0.011 |
| BC1 | Rate constant for biological oxidation of NH4 to NO2 in the reach. | .swq | day-1 | 0.954 | 0.100 | 0.100 | 0.100 |
| BC2 | Rate constant for biological oxidation of NO2 to NO3 in the reach. | .swq | day-1 | 1.775 | 0.200 | 0.200 | 0.200 |
| BC3 | Rate constant for hydrolysis of organic N to NH4 in the reach. | .swq | day-1 | 0.013 | 0.167 | 0.012 | 0.008 |
| BC4 | Rate constant for mineralization of organic P to dissolved P in the reach. | .swq | day-1 | 0.406 | 0.194 | 0.117 | 0.010 |
| CANMX | Maximum canopy index. | .hru | mm | 0.000 | 9.468 | 4.647 | 1.661 |
| CDN | Denitrification exponential rate coefficient. | .bsn | - | 1.103 | 1.120 | 1.120 | 3.000 |
| CH_COV(1) | Channel erodibility factor. | .rte | - | 0.108 | 0.681 | 0.953 | 0.673 |
| CH_COV(2) | Channel cover factor. | .rte | - | 0.130 | 0.375 | 0.024 | 0.028 |
| CH_KI | Effective hydraulic conductivity in tributary channel alluvium. | .sub | mm hr-1 | 1.651 | 148.900 | 58.540 | 146.600 |
| CH_KII | Effective hydraulic conductivity in main channel alluvium. | .rte | mm hr-1 | 10.540 | 0.025 | 3.969 | 0.938 |
| CH_NI | Manning's n value for tributary channels. | .sub | - | 0.014 | 0.014 | 0.014 | 0.098 |
| CH_NII | Manning’s n value for the main channel. | .rte | - | 0.147 | 0.111 | 0.145 | 0.057 |
| CMN | Rate factor for humus mineralization of active organic nutrients. | .bsn | - | 0.001 | 0.000 | 0.000 | 0.002 |
| CN_F | Initial SCS runoff curve number for moisture condition II. | .mgt | % | 0.242 | -0.237 | -0.082 | -0.130 |
| DEP_IMP | Depth to impervious layer in soil profile (mm). | .hru | mm | 0 | 0 | 0 | 1656 |
| ERORGN | Organic N enrichment ratio. | .hru | - | 0.000 | 5.000 | 4.087 | 1.652 |
| ERORGP | Organic P enrichment ratio. | .hru | - | 0.000 | 1.880 | 3.286 | 0.000 |
| ESCO | Soil evaporation compensation factor. | .hru | - | 0.010 | 0.978 | 0.996 | 0.309 |
| GWQMN | Threshold depth of water in the shallow aquifer required for return flow to occur. | .gw | mm H2O | 0 | 50 | 109 | 4669 |
| GWSOLP | Soluble P concentration in groundwater flow. | .gw | mg P L-1 | 0.055 | 0.057 | 0.057 | 0.077 |
| HLIFE_ NGW | Half-life of nitrate in the shallow aquifer. | .gw | days | 365.25 | 365.25 | 365.25 | 291.60 |
| # Supplementary Material, Table 1, continued. |  |  |  |  |  |  |  |
| Name | Description | File | Units | North Fork | South Fork | Sycan | Main |
| ICN | Curve number calculation method. | .bsn | - | 0 | 0 | 1 | 0 |
| LABP | Initial soluble P concentration in soil layer. | .chm | mg P/kg soil | 108.000 | 7.224 | 57.940 | 0.000 |
| NPERCO | Nitrate percolation coefficient. | .bsn | - | 0.258 | 0.085 | 0.085 | 0.010 |
| P_N | Algal preference factor for ammonia. | .wwq | - | 0.358 | 0.677 | 0.890 | 0.010 |
| PHOSKD | P soil partitioning coefficient. | .bsn | m3 Mg-1 | 150 | 165 | 132 | 100 |
| PLAPS | Precipitation lapse rate. | .sub | mm H2O km-1 | 698.400 | 708.100 | 699.500 | 712.700 |
| PPERCO | P percolation coefficient. | .bsn | 10 m3 Mg-1 | 16.310 | 12.210 | 11.120 | 10.000 |
| PRF | Peak rate adjustment factor for sediment routing in the main channel. | .bsn | - | 0.812 | 1.908 | 0.690 | 1.666 |
| RCHRG_DP | Deep aquifer percolation fraction. | .gw | - | 0.000 | 0.246 | 0.825 | 0.009 |
| RHOQ | Algal respiration rate. | .wwq | - | 0.050 | 0.050 | 0.500 | 0.050 |
| RS1 | Local algal settling rate in the reach. | .swq | m day-1 | 0.327 | 1.820 | 1.492 | 0.150 |
| RS2 | Sediment source rate for dissolved P in the reach. | .swq | mg P m-2 day-1 | 3.012 | 0.578 | 0.246 | 0.550 |
| RS3 | Benthic source rate for NH4-N in the reach. | .swq | mg N m-2 day-1 | 0.427 | 0.413 | 0.346 | 0.060 |
| RS4 | Rate coefficient for organic N settling in the reach. | .swq | day-1 | 0.058 | 0.050 | 0.061 | 0.050 |
| RS5 | Organic P settling rate in the reach. | .swq | day-1 | 0.964 | 0.249 | 0.202 | 0.010 |
| RSDCO | Residue decomposition coefficient. | .bsn | - | 0.599 | 0.100 | 0.319 | 0.109 |
| RSDIN | Initial residue cover. | .hru | kg · ha-1 | 2661 | 10000 | 819 | 111 |
| SDNCO | Denitrification threshold water content. | .bsn | - | 0.100 | 0.247 | 0.247 | 0.142 |
| SFTMP | Snowfall temperature. | .bsn | °C | 2.079 | 2.661 | 2.429 | 2.226 |
| SLSUBBSN | Average slope length. | .hru | m | 30.680 | 143.400 | 148.700 | 82.980 |
| SMFMN | Melt factor for snow on December 21. | .bsn | mm · °C-1 · day-1 | 1.838 | 1.800 | 2.052 | 4.022 |
| SMFMX | Melt factor for snow on June 21. | .bsn | mm · °C-1 · day-1 | 1.838 | 1.800 | 3.500 | 4.022 |
| SMTMP | Snow melt base temperature. | .bsn | °C | 4.803 | -1.241 | -2.710 | -3.392 |
| SNO50COV | Fraction of snow volume represented by SNOCOVMX corresponding to 50% snow cover. | .bsn | - | 0.307 | 0.010 | 0.180 | 0.412 |
| SNOCOV-MX | Minimum snow water content corresponding to 100% snow cover. | .bsn | mm | 296.200 | 1.000 | 533.300 | 220.600 |
|  | Supplementary Material, Table 1, concluded. |  |  |  |  |  |  |
| Name | Description | File | Units | North Fork | South Fork | Sycan | Main |
| SOL_AWC | Available water capacity of the soil layer. | .sol | fraction | -0.053 | -0.194 | -0.239 | 0.115 |
| SOL_K | Saturated hydraulic conductivity. | .sol | fraction | -0.220 | -0.199 | 0.019 | -0.167 |
| SOL_ORGN | Initial organic N concentration in the soil layer. | .chm | mg N/kg soil | 1897 | 1 | 1717 | 0 |
| SOL_ORGP | Initial organic P concentration in the soil layer. | .chm | mg P/kg soil | 1.00 | 63.06 | 99.62 | 500.00 |
| SOL_Z | Depth from soil surface to bottom of layer. | .sol | fraction | 0.034 | -0.120 | 0.084 | 0.222 |
| SOLN | Initial NO3- concentration in the soil layer. | .chm | mg N/kg soil | 149.300 | 6.971 | 174.000 | 1.000 |
| SPCON | Linear parameter for calculating maximum sediment reentrainment. | .bsn | - | 0.000 | 0.009 | 0.005 | 0.002 |
| SPEXP | Exponent parameter for calculating sediment reentrainment. | .bsn | - | 1.143 | 1.808 | 1.976 | 1.769 |
| SURLAG | Surface runoff lag coefficient. | .bsn | day | 4.462 | 9.893 | 10.580 | 11.960 |
| TIMP | Snow pack temperature lag factor. | .bsn | - | 0.289 | 0.295 | 0.295 | 0.953 |
| TLAPS | Temperature lapse rate. | .sub | °C km-1 | -3.508 | -3.561 | -3.563 | -3.516 |
| USLE_C | USLE equation cropping practices factor. | crop.dat | fraction | 0.000 | -0.216 | -0.078 | -0.040 |
| USLE_K | USLE equation soil erodibility (K) factor. | .sol | fraction | -0.026 | -0.332 | -0.371 | 0.449 |