Review

Flexural behaviour of selected plants under static load

General comments

The paper is surely interesting.

The desiderable characteristics of the plants for soil-bioengineering purposes should be better outlined referrring to elasticity (bank protection and decreasing roughness and flow resistance after bending, not only bank stabilization and/or lever effect, cfr. P1472 L16), plasticity (permanent deformation, survival, etc.), and breaking (survival after “natural” coppicing or pruning).

According to this:

- TITLE: suggestion for this Journal: Flexural behaviour of selected riparian plants under static load

- P1469 L16-26: suggestion: please, move to Introduction section

Statistical tests are missing.

Authors incurred in a systematic mistake in the three points loading test, due to the "flexibility" of the green stems: the geometrical configuration of the test (as stated by DIN 52186 [Tests on wood. Bending test]) is no more valid when the stem is going to be bent so much that ray of the curved stem in the centre is lower or equal to the ray of the semi-cylindrical head of loading. In this situation, when the loads are high enough, the geometry of the test is completely lost, as well shown in the snapshot of the text:

1) bending moment is unknown because, for the shifting of the stem on the supports, the "lever arm" is changing (bending moment Mf = l/2 x F/2, actual l is shortening with load increasing),

2) direction of the forces are no more vertical,

3) the contact points (on the shifted stems), are four and no more three

So in this situation it is not possible to know the true stress on the stem.

This systematic error is increasing with the slope of the stem and with the growing of the load, but it can change, stem by stem, due to the combination of load, modulus of elasticity, slope, etc.

The recommendation is: Authors have to rewrite the text, pointing out that some important assumptions due to the lost of the three point loading geometry have been done and specifying all the effects on the other results. Differently, if the Authors think that the effect of the loosing geometry is negligible, they have to declare it explicitly, explaining and demonstrating why it is not significant.

The proof-reading by a native speaker and the translation of  some technical words are recommended.

The check of quotations and references in the biography is recommended.

A list of symbols and the check of all the formula and units of measurement are suggested (also in tables, figures and captions).

Considering all of the above, as well as the following specific remarks and other numerous minor deficiencies (of which I have also a more detailed list and will present them upon your request), I think that this manuscript is suitable for publication after major revisions.

Specific comments

| Position (page, line) | Errata | Corrige / comments |
| --- | --- | --- |
| P1460 L9 | Fifty | 50; enough? |
| P1460 L13 and P1472 L5-8 | new “value” | “new” sounds a little bit ambitious; “parameter” not “value” |
| P1460 L15 | engineering practival experience | engineering practical experience |
| P1460 LL21 and 23, P1472 L11 and L20: | “river banks”, “riverbanks” and “fluvial slopes” | “river banks”, not “riverbanks” and not “fluvial slopes” (if it is synonymous and the meaning is cross sectional and not longitudinal) |
| P 1461 L 14 and References: |  | Righetti |
| P 1461 L 13-15: |  | chronological or alphabetical order |
| P1462 L14 and Table 1: |  | 50 samples in 6 diameter classes: only 8 for each class? |
| P1462 L19 and 20 | The setup of the bending tests is based on the DIN standard (DIN 52186) for 3 point loading tests. | It is no true at high load (see P1467 L11 below). We have some important methodology mistake (e.g. strength at the rupture limit) due to the false geometry of the test. |
| P 1464 L10-17: | Felast "l" and "I | wrong formula, check for “Felast”, “felast”, “l” and “I” and dimensions |
| P1465 L4 |  = f 12 d / l² | Probably 6, not 12. |
| P1466 L15 | Editing | Return after [g] |
| P1466 L15 | "and the basic apparent speciﬁc weight of the wood" | probably "Basic density" |
| P1466 LL16, 17 |  | as consequence of 1466 15 b |
| P1466 LL17, 20 |  = mo /V and V volume of wood [cm3] | the moisture content of the piece of wood, which weight and volume have been measured, must be zero for Mass and green (more than 30%) for Volume |
| P1466 LL17, 20 |  = mo /V and V volume of wood [cm3] | Vw (w is for wet, U >30% = green stem) |
| P1467 L7-11 P1467 L7: | ... angle of flexibility ... | there is a basic problem in this kind of test, due to the high plasticity of green stem: at high loads the geometry of the test is no more correct and the true stress (and also breaking stress) is not known. We can say that it is a methodological mistake. The final angles are influenced by geometry of the test. Better “parameter” than “value” |
| P1467 L22-23: |  | Repetition: basic – basic |
| P1468 L1 |  | The first statement (relation between diameter and basic weight ) has no graph |
| P1468 L18 |  | It seems strange (Fig. 3) a linear relation with intercept ≠ 0 instead of a power curve with intercept = 0. |
| P1468 L21 | ...the most fragile. | probably "...the most weak" because in the materials behaviour defined "fragile" is another thing. |
| P1468 L28 P1469 L1 | The calculated moduli of elasticity are comparatively lower than .... of five European species | no consideration about the relationship between E and density (or basic density). Obviously density of 5 European species could be so much higher than species studied.... or not! |
| P1469 L6-9 |  | Statistical tests are missing |
| P1469 L6 |  | cfr. with relationship E[Mpa]=696/d [mm] for roots (Fan&Su, 2008 and Operstein,& Frydman, 2000 in Schwarz et al., ESPL 2009) |
| P1469 L6 | The modulus of elasticity declines with the increase in diameter. | No consideration about the possibility that in this research cork + bast contributed to elasticity (as cited in 1468 line 14). This contribution can be less and less significant as the diameter decreases |
| P1469 L16-26: |  | To be moved to Introduction section |
| P1470 | Angle of flexibility | f and angle fleibility are explicitly related. Due to the geometry of the test these values can not be considered correct because there is a methodical error, varying with diameters. So can be use only as a qualitative indication. |
| P1470 L8-10: | relationship |  |
| P1470 L1-13: |  | To be clarified |
| P1470 L15: | textit | ? |
| P1470 and Fig 5 |  | Statistical test missing |
| P1471 L13-15: |  | not above? |
| P1472 L2 | Fig. 9 | figure 9 is lost |
| P1472 L11 and L20: |  | river banks? |
| P1472 L16: |  | most important flow resistance, erosion protection, river bank stabilization |
| P1472 L18 and L24: |  | branch pruning and trunk or stem coppicing |
| P1472 L27: |  | practice? |
| P1473 L1 |  | Declare that a dstributed load test should be preferable |
| Table 1 |  | Statistical test missing! Add e.g. standard error and not only R2. |
| Fig 7 |  | Diameter/age correlation is not linear. |
| Fig. 9 |  | Missing |